#### (10) Bank/Channel Programming

(Downloadable file available upon request)

Note:	Below is my personal Bank/Channel programming choices.

Bank 0 = Ham radio - 6m, 2m, 70cm repeaters and simplex freqs

Bank 1 = UCAN (Public service - Trunked-open (all ID's - all traffic) 800mhz...

Bank 2 = UCAN (Public service - Trunked-closed (only selected ID's, emergency, fire, etc)

Bank 3 = old Public service freqs - UHP and EMS (150 mhz) - simulcast on 800 mhz

Bank 4 = UTA, Trax and Weber Co schools

Bank 5 = Ogden Airport, Hafb Air traffic support, Search/Rescue, etc...

Bank 6 = SLC Int airport (Air, Gnd, Tower, etc)

Bank 7 = Ogden rail road switching yard and train support dispatch

Bank 8 = Cordless telephone and Baby monitors (46 - 49 Mhz) - illegal to listen to these freqs

Bank 9 = experiment/play around - channel

#### (11) Pro-95 Programming Tips

### Pro-95 Scanner ... Tip #1

Trunk Tracking - Exclusive Talk-Group Monitoring

While (S)canning a Trunked public service band (such as our Bank "1" or Bank "2") and you hear some traffic that sounds particularly interesting may want to monitor it exclusively.

PRESS the red TRUNK button (upper right corner of key pad) and HOLD for at least 1 second.

The currently active talk-group will lock-on and traffic from all other talk-groups will be ignored.

Talk-group Text-ID is displayed on the bottom (4th) line of the LCD display at all times.

To return to scanning all talk-groups simply press (S)can button (upper left corner of key pad).

### Pro-95 Scanner ... Tip #2

Trunk Tracking - UCAN (Utah Communications Agency Network ) – N. Utah Public Service

My programming has the Trunked Tracking (UCAN) channels on Bank #1 and Bank #2.

Bank #1 is Open-mode, which means ALL Talk-Groups are monitored.

Bank #2 is Closed-mode, which means only selected Talk-Groups will be monitored.

The Talk-Groups I selected for monitoring are as follows: (the interesting stuff)

All Emergency, EMS, Fire, Police, Swat Team, Strike Force, Police car-to-car, Air Medical, Hospital ER

Things NOT monitored include:

Dispatch & service channels, dog catcher, water meter readers, garbage trucks, road plows, etc...

### Pro-95 Scanner ... Tip #3

Trunk Tracking - UCAN outside of Weber County

My programming has the Trunked Tracking (UCAN) channels on Bank #1 and Bank #2.

The default channels monitored are those used in Weber County, however the total UCAN system included services for most of Northern Utah from Utah county in the south to Box Elder county in the north and from Tooele county in the west to Evanston Wy in the east.

If traveling outside of Weber County you will need to enable some of the other channels as desired. To do this do the following...

Enable scanning for bank #1 by pressing (S)can and then press the #1 button on the keypad.

Disable scanning of any other banks at this time by pressing the #n for any bank "n" that may have been previously enabled.

Pro-95 Scanner ... Tip #3 (continued)

Press (S)can button (upper left of key pad)

Display should show S101 ... if any channels are currently active or *1******** if scanning for bank #1 activity.

Press (M)anual button (upper center of key pad)

Press Up Arrow and/or Down Arrow key (next to pause and mode buttons) to view specific channel numbers.

Note LCD display line #1, upper left ... Mxyy (where M=manual mode, X=Bank#, yy=channel #)

M101 = Weber County

M102 = Mt Ogden (Weber Co + Davis Co + Morgan Co + more)

M103 = Promontory Point (BoxElder + more)

M104 = Francis Peak = (Davis Co + more)

M105 = Davis County

M106 = Layton Utah

M107 ... M30 (see for yourself)

To enable another channel (county/city/etc) Arrow to the desired channel.

Press L/OUT (middle bottom of keypad) to toggle lock/un-lock the desired channel.

Notice LCD display line #1: M101~**DL+MO

The "L" in position #9 = LockOut or "*" in position #9 = NOT locked out

Proceed to Unlock the channel that corresponds to your scanning location.

You may want to Lock-out the channels that correspond to any other locations to speed up scanning.

Press (S)can to commence scanning of the desired UCAN location.

Note: UCAN County/Location Text-ID displays on LCD line #3 when any traffic exists.

### Pro-95 Scanner ... Tip #4

LCD Display symbols definitions ...

LCD display line #1:

Example: Mbcc~**DL+MO

Position 1-4 (Mbcc): Mode = (M)anual or (S)can, b=Bank #, cc=Channel #)

Position 5: (~) = squelch is open, (*) squelch is closed

Position 8: (D) = Control channel, Data decode

Position 9: (L) Locked Out or (*) Un-locked ---- if manual mode

Position 9: (^) Scanning upward or (v) scanning down ward ---- if scanning mode

Position 10: (+) Trunked open mode (scan all talk groups)

Position 10: (-) Trunked closed mode (scan only selected talk groups)

Position 11-12: MO or MOT = Motorola Trunked Mode

LCD display line #2:

Example: VC Mmm.Kkkkk

(VC) = Voice Channel or ( ) = Control channel - data

(Mmm) = Mhz frequency

(Kkkkk) = Khz frequency

LCD display line #3:

Example: Weber County

(UCAN Data Channel - Location)

LCD display line #4:

Example: WC Fire Op 2

Example: ID 12345

Example: MOT 67890

Talk Group Text ID (if known ID and text programmed)

Talk Group ID ## (if known ID but not text programmed)

Talk Group MOT ## (if unknown ID)

END